VALEW QUALITY TRUCK BODIES

12522 Violet Rd. Adelanto, Ca. 92301

(760-246-4878)

Water Truck Operations

- When spraying water always let the engine RPM’s start to wind down before shutting the spray valves. This will help to prevent water hammer, which can break the pump shaft.

- When engaging the PTO, come to a complete stop, and then engage the PTO. Note to make sure that the transmission is in the manual mode in 1st gear for best performance.

- The spray system should be operated between 1500 - 2200 RPM’s

- The hose reel should be operated between 1000 - 1400 RPM’s.

- Never leave the PTO engaged when the tank is empty. It will break the water pump shaft if left on with no water going through the pump.

- Always be aware of the surroundings when spraying water.